IT5611

Embedded Systems and Internet of Things Laboratory

A Practical Record

Submitted By

Yuvaraj K

2022506079

B. Tech Information Technology (6/8)

Department of Information Technology

Madras Institute of Technology

Anna University, Chennai – 600 044

January – May 2025

Bonafide Certificate

| Name | : Yuvaraj K |
| --- | --- |
| Register No. | : 2022506079 |
| Subject | : IT5612 Embedded Systems and Internet of Things Laboratory |
| Department | : Information Technology |

Certified to the bonafide record of practical work done by Mr. Yuvaraj K in the IT5611 Embedded Systems and Internet of Things Laboratory during the period January 2025 to May 2025.

Date:

Submitted for the Practical Examination held on: 28/04/2025

Dr. S. Umamaheswari

Staff In Charge

Table of Contents

| Exp No. | Date | Title of the Experiment | Page No. | Signature |
| --- | --- | --- | --- | --- |
| 1 | 27/01/25 | Study of 8051 Microcontroller | 1 |  |
| 2 | 27/01/25 | Implementation of Arithmetic Operations using 8051 Microcontroller - N-number Addition - Multibyte Addition - Multibyte Subtraction - 16-bit and 8-bit Multiplication - 8-bit Division | 22 |  |
| 3 | 03/02/25 | Implementation of Array-Based Programs using 8051 Microcontroller - Reversing a String - Palindrome - Largest in Array - Smallest in Array - Bubble Sort | 34 |  |
| 4 | 10/02/25 | Programming I/O Ports - Toggle LEDs with Arbitrary Delay - Toggle LEDs with 1 s Delay - Toggle LEDs with 1 s Delay by Timer - Read Switches’ Status, Write on LEDs - Read Switch and Reflect on LED | 46 |  |

| Exp No. | Date | Title of the Experiment | Page No. | Signature |
| --- | --- | --- | --- | --- |
| 5 | 17/02/25 | Waveform Generation - Triangle Wave of 2 KHz Frequency - Sawtooth Wave of 2 KHz Frequency - Sine Wave of Arbitrary Frequency - Count Pulses and Reflect on LEDs | 62 |  |
| 6 | 24/02/25 | Serial Communication and Interrupt Handling - Toggle LEDs with 1 s Delay and Generate Square Wave of 75% Duty Cycle using Interrupts - Continuously Transmit Message - Transmit and Receive – No Serial - Transmit and Receive using Serial Communication | 76 |  |
| 7 | 10/03/25 | 8051 Programming in Embedded C - Toggle LEDs with Arbitrary S/W Delay - Toggle LEDs with 1 s Delay by Timer - Read Switches’ Status, Write on LEDs - Read Switch, Reflect on LED - Triangle Wave of 2 KHz Frequency - Sawtooth Wave of 2 KHz Frequency - Sine Wave of Arbitrary Frequency - Count Pulses and Reflect on LEDs - Toggle LEDs with 1 s Delay and Generate Square Wave of 75% Duty Cycle using Interrupts - Continuously Transmit Message - Transmit and Receive using Serial Communication | 94 |  |

| Exp No. | Date | Title of the Experiment | Page No. | Signature |
| --- | --- | --- | --- | --- |
| 8 | 10/03/25 | Programming for Accessing External Memory Data - External Memory and Internal Memory Data Exchange in Assembly - External Memory and Internal Memory Data Exchange in C | 122 |  |
| 9 | 17/03/25 | Basic Arduino Programming for Digital I/O Toggle LED with 1 s Delay Read and Transmit Serially Read Switch, Reflect on LED Traffic Signal | 127 |  |
| 10 | 24/03/25 | Arduino Programming for Development of IoT Applications Box Junction Thermistor + Buzzer Ultrasonic Sensor + LED IR Sensor + Servo Motor Motion Detector | 139 |  |
| 11 | 21/04/25 | Development of IoT Applications using Wireless Technologies Wi-Fi Bluetooth GPS | 157 |  |

Toggle LEDs using Arbitrary Software Delay

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
|  |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
| 100 | LOOP: | MOV P1, #0FFH | ; toggle LED on |
| 103 | ON: | ACALL DELAY | ; call delay routine |
| 105 |  | MOV P1, #0H | ; toggle LED off |
| 108 | OFF: | ACALL DELAY | ; |
| 10A |  | SJMP LOOP | ; continue toggling |
|  |  | ORG 200H |  |
| 200 | DELAY: | MOV R1, #02H | ; arbitrary value 1 |
| 202 | DLY1: | MOV R2, #0B4H | ; arbitrary value 2 |
| 204 | DLY2: | MOV R3, #0FFH | ; arbitrary value 3 |
| 206 | DLY3: | DJNZ R3, DLY3 | ; innermost loop |
| 208 |  | DJNZ R2, DLY2 | ; inner loop |
| 20A |  | DJNZ R1, DLY1 | ; outer loop |
| 20C |  | RET | ; end of subroutine |
|  |  | END |  |

Output

Toggle LEDs using 1 Second Software Delay

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
|  |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
| 100 | LOOP: | MOV P1, #0FFH | ; toggle LED on |
| 103 | ON: | ACALL DELAY | ; 1s delay |
| 105 |  | MOV P1, #0H | ; toggle LED off |
| 108 | OFF: | ACALL DELAY | ; 1s delay |
| 10A |  | SJMP LOOP | ; continue toggling |
|  |  | ORG 200H |  |
| 200 | DELAY: | MOV R1, #14H | ; to repeat 0.5s delay 20 times |
| 202 | DLY1: | MOV R2, #5AH | ; inner loop counter |
| 204 | DLY2: | MOV R3, #0FFH | ; innermost loop counter |
| 206 | DLY3: | DJNZ R3, DLY3 | ; innermost loop |
| 208 |  | DJNZ R2, DLY2 | ; inner loop |
| 20A |  | DJNZ R1, DLY1 | ; outer loop |
| 20C |  | RET | ; end of subroutine |
|  |  | END |  |

Output

Toggle LEDs using 1 Second Timer Delay

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
|  |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #01H | ; set timer 0 in mode 1 – 16-bit timer |
| 103 | LOOP: | MOV P1, #0FFH | ; toggle LED on |
| 106 | ON: | ACALL DELAY | ; 1s delay routine |
| 108 |  | MOV P1, #0H | ; toggle LED off |
| 10B | OFF: | ACALL DELAY | ; |
| 10D |  | SJMP LOOP | ; continue toggling |
|  |  | ORG 200H |  |
| 200 | DELAY: | MOV R1, #14H | ; to run 0.05s delay 20 times |
| 202 | DLY: | MOV TLO, #0FDH | ; lower byte of 16-bit timer |
| 205 |  | MOV THO, 46H | ; upper byte of 16-bit timer |
| 208 |  | SETB TR0 | ; start timer 0 |
| 20A | CHECK: | JNB TF0, CHECK | ; monitor timer 0’s flag |
| 20D | STOP: | CLR TF0 | ; clear timer flag |
| 20F |  | CLR TR0 | ; end timer 0 |
| 211 |  | DJNZ R1, DLY | ; repeat 0.05s delay |
| 213 |  | RET | ; end subroutine |
|  |  | END |  |

Output

Read Status of Switches, Write to LEDs

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
|  |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV P2, #0FFH | ; configure port 2 as input |
| 103 | LOOP: | MOV P1, P2 | ; write status of port 2 switches to port 1 LEDs |
| 106 |  | SJMP LOOP | ; continue reading status of switches |
|  |  | END |  |

Input 1					Input 2

Output 1					Output 2

Read Status of Switch, Write to LED

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
|  |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
|  |  | MOV P3, #0FFH | ; configure port 2 as input |
|  | LOOP: | JB P3.2, SETBIT | ; check status of switch in port 3 |
|  |  | CLR P1.3 | ; reflect switch off status in port 1 LED |
|  |  | SJMP HERE | ; |
|  | SETBIT: | SETB P1.3 | ; reflect switch on status in port 1 LED |
|  | HERE: | SJMP LOOP | ; continue reading status of switch |
|  |  | END |  |

Input 1					Input 2

Output 1					Output 2

Triangle Wave – 2kHz Frequency

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #02H | ; configure timer for wave generation |
| 103 |  | MOV TH0, #19H | ; value required to generate 0.25 ms delay |
| 106 | HERE: | MOV P1, #00H | ; |
| 109 |  | CLR A | ; indicates rise |
| 10A |  | ACALL WAVE | ; rise part of wave |
| 10C |  | INC A | ; indicates fall |
| 10D |  | ACALL WAVE | ; fall part of wave |
| 10F |  | SJMP HERE | ; |
|  |  | ORG 200H |  |
| 200 | WAVE: | SETB TR0 | ; subroutine |
| 202 |  | JNZ FALL | ; |
| 204 | RISE: | INC P1 | ; rise part, only increments |
| 206 |  | JNB TF0, RISE | ; |
| 209 |  | SJMP ENDT | ; |
| 20B | FALL: | DEC P1 | ; fall part, only decrements |
| 20D |  | JNB TF0, FALL | ; |
| 210 | ENDT: | CLR TF0 | ; |
| 212 |  | CLR TR0 | ; |
| 214 |  | RET |  |
|  |  | END |  |

Output

Sawtooth Wave – 2kHz Frequency

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #01H | ; configure timer for wave generation |
| 103 |  | MOV P1, #00H | ; |
| 106 |  | CLR A | ; indicates rise |
| 107 | HERE: | ACALL WAVE | ; rise part of wave |
| 109 |  | INC A | ; indicates fall |
| 10A |  | ACALL WAVE | ; fall part of wave |
| 10C |  | SJMP HERE | ; |
|  |  | ORG 200H |  |
| 200 | WAVE: | MOV TH0, #0FEH | ; TH value for 0.5 ms delay |
| 203 |  | MOV TL0, #33H | ; TL value for 0.5 ms delay |
| 206 |  | SETB TR0 | ; |
| 208 |  | JNZ FALL | ; |
| 20A | RISE: | INC P1 | ; rise part, only increments |
| 20C |  | JNB TF0, RISE | ; |
| 20F |  | SJMP ENDT | ; |
| 211 | FALL: | MOV P1, #00H; | ; fall part, instantly goes down to 0 |
| 214 | ENDT: | CLR TF0 | ; |
| 216 |  | CLR TR0 | ; |
| 218 |  | RET | ; |
|  |  | END | ; |

Output

Sine Wave – Arbitrary Frequency

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
| 100 | CYCLE: | MOV DPTR, #DACIN | ; location of lookup table of sine values |
| 103 |  | MOV R1, #4AH | ; no. of sine values |
| 105 | LOOP: | CLR A | ; |
| 106 |  | MOVC A, @A+DPTR | ; current sine value copied to Acc |
| 107 |  | MOV P1, A | ; generating the sine wave |
| 109 |  | INC DPTR | ; |
| 10A |  | DJNZ R1, LOOP | ; |
| 10C |  | SJMP CYCLE | ; |
|  |  | ORG 200H |  |
|  | DACIN: | DB 127, 138, 149, 160, 170, 180, 190, 200, 209, 217, 224, 231, 237 |  |
|  |  | DB 242, 246, 250, 252, 254, 255, 254, 252, 250, 246, 242, 237, 231 |  |
|  |  | DB 224, 217, 209, 200, 190, 180, 170, 160, 149, 138, 127, 116, 105 |  |
|  |  | DB 94, 84, 73, 64, 54, 45, 37, 30, 23, 17, 10, 8, 4, 2, 1, 0, 1, 2, 4, 8 |  |
|  |  | DB 10, 17, 23, 30, 37, 45, 54, 64, 73, 84, 94, 105, 116, 127 |  |
|  |  | END |  |

Output

Count Pulses

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H | ; |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #60H | ; configure counter 1 in mode 2 |
| 103 |  | MOV TH1, #0H | ; initially, counter is 0 |
| 106 |  | SETB P3.5 | ; configure pin T1 to generate clock pulses |
| 108 |  | SETB TR1 | ; start counter |
| 10A | COUNT: | MOV P1, TL1 | ; reflect count value on LEDs |
| 10D |  | SJMP COUNT | ; continue reflecting |
|  |  | END |  |

Input

Non-periodic intervals

Output

Interrupts – Toggle LEDs + Square Wave

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 | MAIN: | MOV TMOD, #01H | ; timer 0 in mode 1 for 0.05 s delay |
| 103 |  | MOV IE, #82H | ; enable timer 0 interrupt |
| 106 |  | MOV TH0, #4BH | ; |
| 109 |  | MOV TL0, #0FDH | ; |
| 10C |  | SETB TR0 | ; |
| 10E |  | MOV R0, #14H | ; counter for LED toggle |
| 110 |  | MOV R1, #2H | ; counter for ‘ON’ part of square wave |
| 112 |  | MOV P1, #0FFH | ; LED on |
| 115 |  | MOV P2, #0FFH | ; |
| 118 |  | SETB P2.7 | ; ‘ON’ part of wave |
| 11A | HERE: | SJMP HERE | ; |
|  |  | ORG 0BH |  |
| 0B |  | DEC R0 | ; LED counter decremented after every 0.05 s |
| 0C |  | ACALL SR | ; subroutine to handle LED and square wave |
| 0E |  | RETI | ; |
|  |  | ORG 150H |  |
| 150 |  | CLR TF0 | ; |
| 152 |  | CLR TR0 | ; |
| 154 |  | MOV A, R0 | ; counter for LED copied to Acc |
| 155 |  | JNZ SQW | ; if counter = 0, toggle LED, else square wave |
| 157 | LED: | JB P1.0, LOFF | ; if LED is ON, turn if OFF, and vice versa |

| 15A | LON: | MOV P1, #0FFH | ; LED is OFF, so toggle it ON |
| --- | --- | --- | --- |
| 15D |  | MOV R0, #14H | ; reinitialise counter for LED toggle |
| 15F |  | SJMP SQW | ; go handle square wave |
| 161 | LOFF: | MOV P1, #0H | ; LED is ON, so toggle it OFF |
| 164 |  | MOV R0, #14H | ; reinitialise counter for LED toggle |
| 166 | SQW: | MOV A, R1 | ; counter for ‘ON’ part of wave copied to Acc |
| 167 |  | JZ OFF | ; if counter = 0, ‘ON’ over, go to ‘OFF’ |
| 169 | ON: | SETB P2.7 | ; ‘ON’ part of wave |
| 16B |  | DEC R1 | ; counter for ‘ON’ part of wave decremented |
| 16C |  | SJMP ST | ; need to restart timer |
| 16E | OFF: | CLR P2.7 | ; ‘OFF’ part of wave |
| 170 |  | MOV R1, #3H | ; reinitialising counter for ‘ON’ part |
| 172 | ST: | MOV TH0, #4BH | ; |
| 175 |  | MOV TL0, #0FDH | ; |
| 178 |  | SETB TR0 | ; |
| 17A |  | RET | ; |
|  |  | END |  |

Output

Every 1 second

Transmit Message Serially

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #20H | ; configure timer for serial communication |
| 103 |  | MOV TH1, #-3 | ; set baud rate of 9600 |
| 106 |  | MOV SCON, #40H | ; enable serial transmission |
| 109 |  | SETB TR1 | ; start baud rate of 9600 |
| 10B | CYCLE: | MOV DPTR, #MSG | ; location of input message |
| 10E | LOOP: | CLR A | ; using Acc as offset, hence setting it to 0 |
| 10F |  | MOVC A, @A+DPTR | ; store current character in Acc |
| 110 |  | INC DPTR | ; go to next character |
| 111 |  | MOV SBUF, A | ; transmit character serially |
| 113 | WAIT: | JNB TI, WAIT | ; wait for transmission to complete |
| 116 |  | CLR TI | ; transmission complete |
| 118 |  | JZ CYCLE | ; if end of string reached, reinitialise |
| 11A |  | SJMP LOOP | ; continue transmitting |
|  |  | ORG 200H |  |
|  | MSG: | DB ‘Welcome to IT Department’, 0 |  |
|  |  | END |  |

Output

Transmit and Receive – without Interrupts

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #20H | ; configure timer for serial communication |
| 103 |  | MOV TH1, #-3 | ; baud rate = 9600 |
| 106 |  | MOV SCON, #50H | ; enable serial transmission and reception |
| 109 |  | MOV P0, #0FFH | ; configure port 0 switches for input |
| 10C |  | SETB TR1 | ; start baud rate of 9600 |
| 10E | TRA: | MOV SBUF, P0 | ; transmit switches reading to SBUF |
| 111 | TWAIT: | JNB TI, TWAIT | ; wait till transmission is complete |
| 114 |  | CLR TI | ; |
| 116 | RWAIT: | JNB RI, RWAIT | ; receiving character from SBUF |
| 119 | REC: | MOV P1, SBUF | ; move received character to LED |
| 11C |  | CLR RI | ; |
| 11E |  | SJMP TRA | ; continue transmission and reception |
|  |  | END |  |

Input

Change the value in Port 0; the value will be transmitted serially to SBUF.

For reception, type a character in the UART window. It will be serially received.

Output

Transmitted characters appear on UART Window.

Received characters are written to Port 1.

Transmit and Receive – with Interrupts

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV TMOD, #20H | ; configure timer for serial communication |
| 103 |  | MOV TH1, #-3 | ; baud rate = 9600 |
| 106 |  | MOV SCON, #50H | ; enable serial transmission and reception |
| 109 |  | MOV IE, #90H | ; enable serial port interrupt |
| 10C |  | MOV P0, #0FFH | ; configure port 0 switches for input |
| 10F |  | SETB TR1 | ; start baud rate of 9600 |
| 111 |  | MOV SBUF, P0 |  |
| 114 | HERE: | SJMP HERE |  |
|  |  | ORG 23H |  |
| 23 | ISRTI: | JNB TI, ISRRI |  |
| 26 |  | ACALL SRTI | ; call transmit subroutine |
| 28 |  | RETI |  |
| 29 | ISRRI: | ACALL SRRI | ; call receive subroutine |
| 2B |  | RETI |  |
|  |  | ORG 200H |  |
| 200 | SRTI: | CLR TI | ; transmission done, so clearing TI |
| 202 |  | MOV SBUF, P0 | ; transmit next character |
| 205 |  | RET | ; |
|  |  | ORG 220H |  |
| 220 | SRRI: | MOV P1, SBUF | ; display serially received character on LED |
| 223 |  | CLR RI | ; reception done, so clear RI |

| 225 |  | RET | ; |
| --- | --- | --- | --- |
|  |  | END | ; |

Input

Change the value in Port 0; the value will be transmitted serially to SBUF.

For reception, type a character in the UART window. It will be serially received.

Output

Transmitted characters appear on UART Window.

Received characters are written to Port 1.

Toggle LEDs with Arbitrary S/W Delay

#include <reg51.h>

void delay() {

unsigned int i, j;

for (i=0; i<267; i++)

for (j=0; j<1275; j++);

}

void main() {

while(1) {

P1 = 0xFF;

delay();

P1 = 0x00;

delay();

}

}

Output

Arbitrary Delay

Toggle LEDs with 1s Delay

#include <reg51.h>

void delay() {

unsigned int i;

for (i=0; i<20; i++) {

TL0 = 0xFD;

TH0 = 0x4B;

TR0 = 1;

while (TF0==0);

TR0 = 0;

TF0 = 0;

}

}

void main() {

TMOD = 0x01;

while (1) {

P1=0xFF;

delay();

P1=0x00;

delay();

}

}

Output

Read Status of Switches, Write to LEDs

#include <reg51.h>

void main() {

P2 = 0xFF;

while (1)

P1 = P2;

}

Input 1					Input 2

Output 1					Output 2

Read Status of Switch, Write to LED

#include <reg51.h>

sbit my_switch = P3^2;

sbit led = P1^3;

void main() {

my_switch=1;

while (1)

led = my_switch;

}

Input 1					Input 2

Output 1					Output 2

Triangle Wave – 2kHz Frequency

#include <reg51.h>

void wave() {

TR0 = 1;

if (P1==0)

while (TF0==0)

P1++;

else

while (TF0==0)

P1--;

TR0 = 0;

TF0 = 0;

}

void main() {

TMOD = 0x02;

TH0 = 0x19;

while (1) {

P1 = 0;

wave();

wave();

}

}

Output

Sawtooth Wave – 2kHz Frequency

#include <reg51.h>

void wave() {

TH0 = 0xFE;

TL0 = 0x33;

TR0 = 1;

while (TF0==0)

P1++;

TR0 = 0;

TF0 = 0;

}

void main() {

TMOD = 0x01;

while (1) {

P1 = 0;

wave();

}

}

Output

Sine Wave – Arbitrary Frequency

#include <reg51.h>

void main() {

unsigned char sin_vals[] = {

127, 138, 149, 160, 170, 180, 190, 200, 209, 217, 224, 231, 237, 242, 246, 250,

252, 254, 255, 254, 252, 250, 246, 242, 237, 231, 224, 217, 209, 200, 190, 180, 170, 160, 149, 138, 127, 116, 105, 94, 84, 73, 64, 54, 45, 37, 30, 23, 17, 10, 8, 4, 2, 1, 0, 1, 2, 4, 8, 10, 17, 23, 30, 37, 45, 54, 64, 73, 84, 94, 105, 116

};

unsigned int i;

while (1)

for (i=0; i<sizeof(sin_vals); i++)

P1 = sin_vals[i];

}

Output

Count Pulses

#include <reg51.h>

sbit pulse = P3^5;

void main() {

TMOD = 0x60;

TH1 = 0x00;

pulse = 1;

TR1 = 1;

while (1)

P1 = TL1;

}

Input

Non-periodic intervals

Output

Interrupts – Toggle LEDs + Square Wave

#include <reg51.h>

sbit pin = P2^7;

unsigned char led_c;

unsigned char wave_c;

void timer0() interrupt 1 {

led_c--;

TR0 = 0;

TF0 = 0;

if (led_c == 0) {

P1 = P1^0xFF;

led_c = 20;

}

if (wave_c == 0) {

pin = 0;

wave_c = 3;

} else {

pin = 1;

wave_c--;

}

TH0 = 0x4B;

TL0 = 0xFD;

TR0 = 1;

}

void main() {

TMOD = 0x01;

IE = 0x82;

TH0 = 0x4B;

TL0 = 0xFD;

led_c = 20;

wave_c = 2;

P1 = 0xFF;

P2 = 0;

pin = 1;

TR0 = 1;

while (1);

}

Output

Every 1 second

Transmit Message Serially

#include <reg51.h>

void main() {

unsigned char message[] = "Welcome to IT Department";

TMOD = 0x20;

TH1 = -3;

SCON = 0x40;

TR1 = 1;

while (1) {

unsigned int i;

for (i=0; i<sizeof(message); i++) {

SBUF = message[i];

while (TI==0);

TI = 0;

}

}

}

Output

Transmit and Receive with Interrupt

#include <reg51.h>

void serial() interrupt 4 {

if (TI==1) {

TI = 0;

SBUF = P0;

} else {

P1 = SBUF;

RI = 0;

}

}

void main() {

TMOD = 0x20;

TH1 = -3;

SCON = 0x50;

IE = 0x90;

P0 = 0xFF;

TR1 = 1;

SBUF = P0;

while (1);

}

Input

Change the value in Port 0; the value will be transmitted serially to SBUF.

For reception, type a character in the UART window. It will be serially received.

Output

Transmitted characters appear on UART Window.

Received characters are written to Port 1.

Toggle LEDs – External Interrupt

#include <reg51.h>

sbit it0 = TCON^0;

sbit int0pin = P3^2;

sbit it1 = TCON^2;

sbit int1pin = P3^3;

void int0() interrupt 0 {

P1 = P1 ^ 0xFF;

}

void int1() interrupt 2 {

P2 = P2 ^ 0xFF;

}

void main() {

IE = 0x85; // IE = 1000 0101 = 85H

it0 = 1;

int0pin = 1;

it1 = 1;

int1pin = 1;

P1 = 0xFF;

P2 = 0xFF;

while (1);

}

Output

Upon triggering P3.3

Upon triggering P3.2

Internal RAM and External RAM Data Exchange – Assembly

| Address | Label | Instruction | Comment |
| --- | --- | --- | --- |
|  |  | ORG 0H |  |
| 0 |  | LJMP 100H |  |
|  |  | ORG 100H |  |
| 100 |  | MOV DPTR, #100H | ; external RAM data starting location |
| 103 |  | MOV R2, #10 | ; no. of bytes of data, acts as counter |
| 105 |  | MOV R0, #40H | ; internal RAM data starting location |
| 107 | LOOP: | MOV A, @R0 | ; internal RAM data to Acc |
| 108 |  | MOV R1, A | ; temporarily store internal RAM data in R1 |
| 109 |  | MOVX A, @DPTR | ; external RAM data to Acc |
| 10A |  | MOV @R0, A | ; external RAM data to internal RAM |
| 10B |  | MOV A, R1 | ; internal RAM data to Acc |
| 10C |  | MOVX @DPTR, A | ; internal RAM data to external RAM |
| 10D |  | INC R0 | ; go to next byte in internal RAM |
| 10E |  | INC DPTR | ; go to next byte in external RAM |
| 10F |  | DJNZ R2, LOOP | ; continue until all 10 bytes are exchanged |
| 111 | HERE: | SJMP HERE |  |
|  |  | END |  |

Input

Output

Internal RAM and External RAM Data Exchange – Embedded C

#include <reg51.h>

#include <absacc.h>

void main() {

unsigned int i;

unsigned char internal[] = "JIHGFEDCBA";

unsigned char val = 'A';

for (i=0; i<10; i++) {

XBYTE[i]=val;

val++;

}

for (i=10; i<20; i++)

XBYTE[i]=internal[i-10];

for (i=0; i<10; i++)

internal[i]=XBYTE[i];

while(1);

}

Input

Output

Exp No: 9

17/03/2025

Basic Arduino Programming for Digital I/O

Objective

To write and execute basic Arduino sketches for

- toggling LED

- serial transmission

- reading switch and reflecting it on LED

- traffic signal simulation

Components Required

| S. No. | Component | Quantity |
| --- | --- | --- |
| 1 | Arduino Mega 2560 | 1 |
| 2 | Power Cord | 1 |
| 3 | Male-to-Male Wires | Required Amount |
| 4 | LED | 3 |

Toggle LEDs – 1s Delay

int led = 13;

void setup() {

pinMode(led, OUTPUT);

digitalWrite(led, LOW);

}

void loop() {

digitalWrite(led, HIGH);

delay(1000);

digitalWrite(led, LOW);

delay(1000);

}

Circuit Diagram

Output

LED is toggled ON and OFF every 1 second.

Aim

To write and execute an Arduino sketch that toggles the LED with a delay of 1 s.

Read and Serial Transmit

int my_switch = 7;

void setup() {

Serial.begin(9600);

pinMode(my_switch, INPUT);

}

void loop() {

if (digitalRead(my_switch)==HIGH)

Serial.write("H");

else

Serial.write("L");

}

Circuit Diagram

| Switch is LOW. | Switch is HIGH. |
| --- | --- |

Output

When switch is LOW, “L” is serially written.

When switch is HIGH, “H” is serially written.

Aim

To write and execute an Arduino sketch that reads the switch and serially transmits a corresponding message.

Read from Switch, Reflect on LED

int my_switch = 7;

int led = 13;

int val = 0;

void setup() {

pinMode(my_switch, INPUT);

pinMode(led, OUTPUT);

}

void loop() {

digitalWrite(led, digitalRead(my_switch));

}

Circuit Diagram

Output

When switch is LOW, LED is OFF.

When switch is HIGH, LED is ON.

Aim

To write and execute an Arduino sketch that reads from a switch and reflects it on the LED.

Traffic Lights

int red = 11;

int yellow = 12;

int green = 13;

void setup() {

pinMode(red, OUTPUT);

pinMode(yellow, OUTPUT);

pinMode(green, OUTPUT);

}

void loop() {

digitalWrite(red, HIGH);

delay(5000);

digitalWrite(red, LOW);

digitalWrite(yellow, HIGH);

delay(2000);

digitalWrite(yellow, LOW);

digitalWrite(green, HIGH);

delay(3000);

digitalWrite(green, LOW);

digitalWrite(yellow, HIGH);

delay(2000);

digitalWrite(yellow, LOW);

}

Aim

To write and execute an Arduino sketch that simulates the working of traffic lights.

Circuit Diagram

Output

Red – 5s

Yellow – 2s

Green – 3s

Yellow – 2s

This sequence is simulated forever, thus simulating traffic lights.

Result

Hence, the following Arduino sketches were successfully implemented.

- Toggle LED

- Read and Transmit Serially

- Switch to LED

- Traffic Signal

Exp No: 10

24/03/2025

Arduino Programming for Development of IoT Applications

Objective

To develop simple IoT applications using Arduino, sensors, and actuators.

Components Required

| S. No. | Component | Quantity |
| --- | --- | --- |
| 1 | Arduino Mega 2560 | 1 |
| 2 | Power Cord | 1 |
| 3 | Male-to-Male Wires | Required Amount |
| 4 | Male-to-Female Wires | Required Amount |
| 5 | LED | 8 |
| 6 | Thermistor | 1 |
| 7 | Ultrasonic Sensor | 1 |
| 8 | IR Sensor | 1 |
| 9 | Servo Motor | 1 |
| 10 | Motion Detector | 1 |

Box Junction

int g1 = 6, r1 = 7, g2 = 8, r2 = 9, g3 = 10, r3 = 11, g4 = 12, r4 = 13;

void setup() {

int pins[] = {g1, r1, g2, r2, g3, r3, g4, r4};

for (int i=0; i<8; i++) {

pinMode(pins[i], OUTPUT);

digitalWrite(pins[i], LOW);

}

}

void loop() {

// 1

digitalWrite(r2, HIGH);

digitalWrite(r3, HIGH);

digitalWrite(r4, HIGH);

digitalWrite(g1, HIGH);

delay(5000);

// 2

digitalWrite(r3, LOW);

digitalWrite(g3, HIGH);

delay(5000);

digitalWrite(g1, LOW);

digitalWrite(g3, LOW);

digitalWrite(r1, HIGH);

digitalWrite(r3, HIGH);

// 3

digitalWrite(r2, LOW);

digitalWrite(g2, HIGH);

delay(5000);

// 4

digitalWrite(r4, LOW);

digitalWrite(g4, HIGH);

delay(5000);

digitalWrite(g2, LOW);

digitalWrite(g4, LOW);

digitalWrite(r2, HIGH);

digitalWrite(r4, HIGH);

// 5

digitalWrite(r3, LOW);

digitalWrite(g3, HIGH);

delay(5000);

digitalWrite(g3, LOW);

Aim

To write and execute an Arduino sketch that simulates the working of traffic lights in a box junction.

digitalWrite(r3, HIGH);

// 6

digitalWrite(r4, LOW);

digitalWrite(g4, HIGH);

delay(5000);

digitalWrite(g4, LOW);

digitalWrite(r4, HIGH);

}

Circuit Diagram

Output

A box junction traffic is simulated. There are 6 possible combinations which are repeated infinitely.

- South is allowed to go straight and right. The others are OFF.

- South and North are allowed to go straight. The others are OFF.

- West is allowed to go straight and right. The others are OFF.

- West and East are allowed to go straight. The others are OFF.

- North is allowed to go straight AND right. The others are OFF.

- East is allowed to go straight AND right. The others are OFF.

Thermistor

float value, temperature;

int thermistor = A0;

int buzzer = 13;

void setup() {

Serial.begin(9600);

pinMode(thermistor, INPUT);

pinMode(buzzer, OUTPUT);

}

void loop() {

value = analogRead(thermistor);

temperature = (value*5000/1024)/10;

Serial.print("Temperature: ");

Serial.println(temperature);

if(temperature >= 30)

digitalWrite(buzzer, HIGH);

else

digitalWrite(buzzer, LOW);

}

Aim

To write and execute an Arduino sketch that activates the buzzer when the temperature exceeds a predefined threshold.

Circuit Diagram

Output

When temperature is below 30°C, buzzer is OFF.

When temperature is equal to or above 30°C, buzzer is ON.

Ultrasonic Sensor

int led = 7, trigger = 8, echo = 9;

float timing = 0.0, distance = 0.0;

void setup() {

pinMode(echo, INPUT);

pinMode(trigger, OUTPUT);

pinMode(led, OUTPUT);

digitalWrite(trigger, LOW);

digitalWrite(led, LOW);

Serial.begin(9600);

}

void loop() {

digitalWrite(trigger, LOW);

delay(2);

digitalWrite(trigger, HIGH);

delay(10);

digitalWrite(trigger, LOW);

timing = pulseIn(echo, HIGH);

distance = (timing*0.034)/2;

Serial.print("Distance: ");

Serial.println(distance);

if (distance <= 50)

digitalWrite(led, HIGH);

else

digitalWrite(led, LOW);

}

Aim

To write and execute an Arduino sketch that activates the LED when the distance from the ultrasonic sensor is below a certain predefined upper limit.

Circuit Diagram

Output

When there is an object at a distance of less than 50 cm from the sensor, LED is ON.

Otherwise, LED is off.

IR Sensor + Servo Motor

#include <Servo.h>

Servo sb;

int ir = 7;

void setup() {

pinMode(ir, INPUT);

sb.attach(8);

sb.write(90);

Serial.begin(9600);

}

void loop() {

if (digitalRead(ir) == HIGH) {

Serial.println("Obstacle!");

sb.write(180);

delay(2000);

sb.write(90);

} else {

sb.write(90);

}

}

Circuit Diagram

Output

The servo motor is initially at the 90° position.

When an object is detected in the proximity of the IR sensor, the servo motor rotates to 180°, stays in that position for 2 s, then goes back to the 90° position.

Otherwise, the servo motor remains at its 90° position.

Aim

To write and execute an Arduino sketch that rotates the servo motor when an object is detected by the infrared sensor.

Motion Detector

int led = 13;

int motion_detector = 12;

void setup() {

pinMode(led, OUTPUT);

pinMode(motion_detector, INPUT);

Serial.begin(9600);

}

void loop() {

if (digitalRead(motion_detector) == HIGH) {

Serial.println("Motion detected!");

digitalWrite(led, HIGH);

} else {

digitalWrite(led, LOW);

}

}

Circuit Diagram

Output

When motion is detected anywhere within 6 m of the PIR sensor, LED is ON.

Otherwise, LED is OFF.

Aim

To write and execute an Arduino sketch that activates the LED upon the detection of motion.

Result

Hence, the following IoT applications using Arduino sketches were successfully implemented.

- Box Junction

- Thermistor + Buzzer

- Ultrasonic + LED

- IR Sensor + Servo Motor

- Motion Detector + Buzzer

Exp No: 11

21/04/2025

Development of IoT Applications using Wireless Technologies

Objective

To develop simple IoT applications by interfacing with different wireless technologies.

Components Required

| S. No. | Component | Quantity |
| --- | --- | --- |
| 1 | NodeMCU | 1 |
| 2 | Power Cord | 1 |
| 3 | HC05 Bluetooth Module | 1 |
| 4 | NEO-6M GPS Receiver Module | 1 |
| 5 | LED | 1 |
| 6 | Resistor – 330 ohms | 1 |
| 7 | Male-to-Female Wires | Required Amount |
| 8 | Female-to-Female Wires | Required Amount |

Wi-Fi

#include <ESP8266WiFi.h>

#include <ESP8266WebServer.h>

const char* ssid = "Team 12";

const char* pw = "anuniyuv";

IPAddress local_ip(192,168,2,1);

IPAddress gateway(192,168,2,1);

IPAddress subnet(255,255,255,0);

ESP8266WebServer server(80);

uint8_t led = D2;

bool status = LOW;

void setup() {

Serial.begin(9600);

pinMode(led, OUTPUT);

WiFi.softAP(ssid, pw);

WiFi.softAPConfig(local_ip, gateway, subnet);

server.on("/", onConnect);

server.on("/ledOn", ledOn);

server.on("/ledOff", ledOff);

server.onNotFound(NotFound);

server.begin();

Serial.println("HTTP server started.");

}

void loop() {

server.handleClient();

if(status)

digitalWrite(led, HIGH);

else

digitalWrite(led, LOW);

}

Aim

To write and execute an Arduino sketch that enables NodeMCU as an HTTP server with Wi-Fi Access Point mode and controls an LED connected at the server, from the client.

void onConnect() {

status = LOW;

server.send(200, "text/html", sendHTML(false));

}

void ledOn() {

status = HIGH;

server.send(200, "text/html", sendHTML(true));

}

void ledOff() {

status = LOW;

server.send(200, "text/html", sendHTML(false));

}

void NotFound(){

server.send(404, "text/plain", "Not found");

}

String sendHTML(uint8_t led){

String html = "" +

"<!DOCTYPE html>\n" +

"<html>\n" +

"<head>\n" +

"<title>LED Control</title>\n" +

"</head>\n" +

"<body>\n" +

"<h1>LED</h1>\n" +

"<form method=\"get\">\n" +

(led ?

"<input type='button' value=\"LED OFF\" onclick=\"window.location.href='/ledOff'\">\n"

:

"<input type=\"button\" value=\"LED ON\" onclick=\"window.location.href='/ledOn'\">\n") +

"</form>\n" +

"</body>\n" +

"</html>\n";

return html;

}

Circuit Diagram

Output

Bluetooth

int led = D4;

void setup() {

pinMode(led, OUTPUT);

Serial.begin(9600);

Serial1.begin(9600);

}

void loop() {

if (Serial1.available()) {

char data;

data = Serial.read();

if (data == '0') {

digitalWrite(led, LOW);

Serial1.println("LED turned OFF.");

} else if (data == '1') {

digitalWrite(led, HIGH);

Serial1.println("LED turned ON.");

} else

Serial1.println("Select either 0 or 1.");

}

}

Aim

To write and execute an Arduino sketch that toggles the LED through commands given via a smartphone that is connected to NodeMCU by Bluetooth.

Circuit Diagram

Output

Select either 0 or 1.

1

LED turned ON.

0

LED turned OFF.

Y

Select either 0 or 1.

1

LED turned ON.

0

LED turned OFF.

GPS

#include <TinyGPS++.h>

#include <SoftwareSerial.h>

SoftwareSerial bt(4, 3);

TinyGPSPlus gps;

float minutes, seconds;

int degree, secs, mins;

void setup() {

Serial.begin(9600);

bt.begin(9600);

}

void loop() {

smartDelay(1000);

double lat, lng, alt;

uint8_t hr, min, sec;

bool loc_v, alt_v, time_v;

lat = gps.location.lat();

loc_v = gps.location.isValid();

lng = gps.location.lng();

alt = gps.altitude.meters();

alt_v = gps.altitude.isValid();

hr = gps.time.hour();

min = gps.time.minute();

sec = gps.time.second();

time_v = gps.time.isValid();

if (!loc_v) {

Serial.println("Latitude:\t*****");

Serial.println("Longitude:\t*****");

} else {

Serial.print("Latitude:\t");

Serial.println(lat, 2);

Serial.print("Longitude:\t");

Serial.println(lng, 2);

DegMinSec(lat);

Serial.print("Latitude in Decimal Degrees: ");

Serial.println(lat, 2);

Serial.print("Latitude in Degrees Minutes Seconds: ");

Serial.print(degree);

Serial.print(" ");

Serial.print(mins);

Aim

To write and execute an Arduino sketch that finds the latitude, longitude, altitude, and time through the GPS receiver module and displays the details on the serial monitor.

Serial.print(" ");

Serial.println(secs);

DegMinSec(lng);

Serial.print("Longitude in Decimal Degrees: ");

Serial.println(lng, 2);

Serial.print("Longitude in Degrees Minutes Seconds: ");

Serial.print(degree);

Serial.print(" ");

Serial.print(mins);

Serial.print(" ");

Serial.println(secs);

}

if (!alt_v)

Serial.println("Altitude:\t*****");

else {

Serial.print("Altitude:\t");

Serial.println(alt, 2);

}

if (!time_v)

Serial.println("Time:\t*****");

else {

char time_string[32];

sprintf(time_string, "Time:\t%02d/%02d/%02d\n", hr, min, sec);

Serial.print(time_string);

}

}

static void smartDelay(unsigned long ms) {

unsigned long start = millis();

do {

while (bt.available())

gps.encode(bt.read());

} while (millis()-start < ms);

}

void DegMinSec(double tot_val)

{

degree = (int)tot_val;

minutes = tot_val - degree;

seconds = 60 * minutes;

minutes = (int)seconds;

mins = (int)minutes;

seconds = seconds - minutes;

seconds = 60 * seconds;

secs = (int)seconds;

}

Circuit Diagram

Output

Result

Hence, IoT applications were successfully developed by interfacing with different wireless technologies such as

- Wi-Fi

- Bluetooth

- GPS